| No. | Question | Answer | Brief Explanation |
| --- | --- | --- | --- |
| 1 | Interface is the communication path between two components. | True | This is the definition of an interface. |
| 2 | A microprocessor accesses memory only for data, not for instructions. | False | It accesses memory for both instructions (fetch) and data. |
| 3 | Memory interfacing is required to read instruction codes and stored data. | True | Memory interfacing enables reading/writing instructions and data. |
| 4 | IO interfacing deals only with internal CPU registers. | False | It deals with external I/O devices, not internal registers. |
| 5 | Keyboard and printer are examples of IO devices. | True | Both are standard input/output peripheral devices. |
| 6 | Interfacing acts as a bridge between user and computer. | True | It connects hardware components for communication. |
| 7 | Interfacing is classified into memory interfacing and IO interfacing. | True | These are the two main types. |
| 8 | The 8085 microprocessor has a 16-bit address bus. | True | 8085 has 16 address lines (A0–A15). |
| 9 | The 8085 can access a maximum of 32 KB memory. | False | 2^16 = 64 KB maximum. |
| 10 | Control signals monitor read and write operations in memory interfacing. | True | RD (read) and WR (write) are control signals. |
| 11 | Memory interfacing requires selection of chip, register, and buffers. | True | Address decoding selects chips; buffers isolate buses. |
| 12 | Memory devices do not require address lines. | False | They need address lines to select specific locations. |
| 13 | Memory interfacing always uses the full 64 KB address space. | False | It uses only the address range required by the design. |
| 14 | EPROM is commonly used as program memory. | True | EPROM is non-volatile and reusable for program storage. |
| 15 | RAM is usually used as data memory. | True | RAM is volatile and used for temporary data. |
| 16 | EPROM and RAM must be placed consecutively in memory. | False | Placement depends on design, not mandatory. |
| 17 | Program memory should start from address 0000H in 8085. | True | Reset vector is at 0000H. |
| 18 | Reset address of 8085 is FFFFH. | False | Reset address is 0000H, not FFFFH. |
| 19 | Memory placement depends on application requirements. | True | Each application has different memory mapping needs. |
| 20 | Memory interfacing is unnecessary in microprocessor systems. | False | It is essential for program and data storage. |
| 21 | Main memory can be directly accessed by the microprocessor. | True | Via address bus, data bus, and control signals. |
| 22 | Secondary memory is faster than main memory. | False | Secondary memory (disk) is much slower. |
| 23 | RAM is volatile memory. | True | RAM loses data when power is turned off. |
| 24 | ROM is non-volatile memory. | True | ROM retains data without power. |
| 25 | Flash memory is an example of NVRAM. | True | Flash is non-volatile RAM. |
| 26 | ROM stores the bootstrap loader program. | True | Boot code is stored in ROM. |
| 27 | PROM can be programmed multiple times. | False | PROM can be programmed only once. |
| 28 | EPROM can be erased using ultraviolet light. | True | UV EPROM requires UV light for erasure. |
| 29 | EEPROM can be erased electronically. | True | Electrically Erasable PROM. |
| 30 | EEPROM stores the retrieved data by system to be saved. | False | Vague/incorrect statement. |
| 31 | DRAM stores data using capacitors. | True | Each DRAM cell uses a capacitor. |
| 32 | DRAM does not require refreshing. | False | DRAM requires periodic refresh. |
| 33 | SRAM is faster than DRAM. | True | SRAM has faster access time. |
| 34 | SRAM is cheaper than DRAM. | False | DRAM is cheaper per bit. |
| 35 | SRAM uses flip-flops to store data. | True | Each SRAM cell is a flip-flop. |
| 36 | Mask ROM is written during manufacturing. | True | Data is masked during fabrication. |
| 37 | PROM is user-programmable only once. | True | One-time programmable. |
| 38 | EPROM was developed to modify PROM contents. | True | EPROM allows erasure and reprogramming. |
| 39 | EPROM write operation is faster than read operation. | False | Writing EPROM is slower than reading. |
| 40 | EPROM can be programmed while mounted on the system board. | False | Needs UV eraser, usually removed. |
| 41 | I/O interfacing uses latches and buffers. | True | Latches and buffers are common in I/O interfacing. |
| 42 | Buffers are used only for output devices. | False | Buffers are used for both input and output. |
| 43 | Latches are mainly used for output interfacing. | True | Latches hold output data stable. |
| 44 | Programmable I/O devices improve system performance. | True | They add flexibility and efficiency. |
| 45 | Intel designed programmable I/O devices for 8085 systems. | True | 8255, 8259, etc., are Intel chips. |
| 46 | 8255 is a programmable parallel interface. | True | Yes, 3 ports, parallel I/O. |
| 47 | 8251 is used for serial communication. | True | USART for serial communication. |
| 48 | 8254 is a programmable timer. | True | Programmable Interval Timer. |
| 49 | 8259 is a programmable interrupt controller. | True | Manages multiple interrupts. |
| 50 | 8279 is a keyboard/display controller. | True | Dedicated for keyboard and display. |
| 51 | Communication between processor and memory is called interfacing. | True | Memory interfacing. |
| 52 | Serial communication transfers all bits simultaneously. | False | Parallel does that. |
| 53 | Parallel communication transfers data bit by bit. | False | Serial does that. |
| 54 | Serial communication sends data one bit at a time. | True | Yes, sequentially. |
| 55 | Parallel communication is faster than serial communication. | True | Multiple bits at once. |
| 56 | Input devices cannot be interfaced with 8085. | False | They can via I/O interfacing. |
| 57 | IO interfacing allows communication with the external world. | True | Connects to peripherals. |
| 58 | Peripheral devices are also called I/O devices. | True | Same meaning. |
| 59 | IO devices communicate directly without interfaces. | False | Need interface chips. |
| 60 | Interface IC helps perform basic CPU functions. | False | Helps I/O, not CPU functions. |
| 61 | Memory-mapped I/O uses the same address space as memory. | True | Shares memory address space. |
| 62 | Memory-mapped I/O requires special IN and OUT instructions. | False | Uses MOV, ADD, etc. |
| 63 | MOV and ADD instructions can work with memory-mapped I/O. | True | Yes, treat I/O as memory. |
| 64 | In memory-mapped I/O, peripherals have separate address space. | False | Same address space as memory. |

PART II: MULTIPLE CHOICE (with Answers & Explanations)

| Q# | Question | Answer | Brief Explanation |
| --- | --- | --- | --- |
| 1 | An Interface is ______ | B) A shared boundary between two components | Definition of interface. |
| 2 | Interfacing provides communication between ______ | C) Two separate components | Connects CPU to memory or I/O. |
| 3 | Interfacing is classified into ______ | B) Memory and I/O | Two main types. |
| 4 | Memory interfacing is required to ______ | B) Access instruction codes and data | To read/write program and data. |
| 5 | I/O interfacing mainly connects ______ | C) Peripheral devices | Keyboard, printer, display, etc. |

| 6 | Keyboard and display are examples of ______ | C) I/O devices | Input/output peripherals. |
| --- | --- | --- | --- |
| 7 | Interfacing circuits should match ______ | B) Microprocessor signals with device requirements | Signal compatibility. |
| 8 | Memory and microprocessor communication needs ______ | C) Read and write signals | RD, WR control signals. |
| 9 | Which is NOT a type of interfacing? | C) Serial | Classification is Memory/I/O. |
| 10 | An 8-bit microprocessor processes: | A) 8 bits at a time | Definition of 8-bit CPU. |
| 11 | Which memory is volatile? | C) RAM | Loses data without power. |
| 12 | Flash memory belongs to: | C) NVRAM | Non-volatile RAM. |
| 13 | Secondary memory includes: | C) Magnetic disks | Hard drives, floppy disks. |
| 14 | Cache memory reduces: | B) Access time | Speeds up CPU memory access. |
| 15 | Peripheral communication can be: | C) Both serial and parallel | Two methods. |
| 16 | Serial communication sends data: | B) Bit by bit | One bit at a time. |
| 17 | Parallel communication sends data: | B) Byte simultaneously | 8 bits at once. |
| 18 | 8279 is used for: | C) Keyboard/display | Dedicated controller. |
| 19 | 8254 is a: | A) Timer | Programmable Interval Timer. |
| 20 | Which Intel IC is used for parallel communication? | B) 8255 | PPI. |
| 21 | Which device is a DMA controller? | A) 8257 | DMA controller. |
| 22 | Which device is a programmable interrupt controller? | 8259 | PIC. |
| 23 | Interfacing enables data transfer between: | C) Processor and peripherals | Main purpose. |
| 24 | Communication between processor and memory is: | B) Interfacing | Memory interfacing. |
| 25 | Peripheral devices are also known as: | B) Input-output devices | Same meaning. |
| 26 | 8085 has address bus width of: | C) 16-bit | A0–A15. |
| 27 | Maximum memory accessible by 8085 is: | 64 KB (not listed? 2^16=64KB) | 64 KB. |
| 84 | A 20-bit address bus can locate ______ | A) 1,048,576 locations | 2^20 = 1,048,576. |
| 85 | Which is not an arithmetic instruction? | D) ROL (rotate left) | Rotate is logical. |
| 86 | During a read operation the CPU fetches ______ | D) All of the above | Instruction, address, data. |
| 87 | Which is not an 8086/8088 general-purpose register? | D) Address segment | Not a GPR. |
| 88 | A 20-bit address bus allows access to memory of capacity ______ | A) 1 MB | 2^20 = 1 MB. |
| 89 | Which microprocessor accepts the program written for 8086 without changes? | D) 8088 | Code-compatible. |
| 90 | Which group of instructions does not affect the flags? | C) Data transfer operations | MOV, etc., don't change flags. |
| 91 | The result of MOV AL, 55 is to store ______ | 55H (store 55H in AL) | Immediate value 55H. |
| 138 | A T-state refers to: | C) One clock period | Basic timing unit. |
| 139 | A machine cycle is defined as: | B) Time required to access memory or I/O | One memory/IO operation. |
| 140 | Which signal is used to read data from memory or I/O? | C) RD | Read signal. |
| 141 | Which best defines a microcontroller? | B) A silicon chip that includes microprocessor, memory and I/O in one package | Definition. |
| 142 | An 8-bit microprocessor processes: | A) 8 bits at a time | Data bus width. |
| 143 | Which memory type stores information that does not change? | B) EPROM | Non-volatile program memory. |
| 144 | In the fetch-decode-execute cycle, the step that interrupts the instruction is: | C) Execute | Interrupts checked after each instruction. |
| 145 | Which operation is NOT part of the ALU's functions? | D) Render HTML | Not hardware operation. |
| 146 | Which assembly mnemonic corresponds to hexadecimal opcode 3C in 8085? | B) INR A | 3C = INR A. |

PART III: WORKOUT & SHORT ANSWER (with Answers)

Question 1: Find memory address from CS:IP (real mode)

Formula: Physical Address = (CS × 10H) + IP

| (a) CS=1A00H, IP=B800H | (b) CS=2300H, IP=1000H | (c) CS=2300H, IP=1A00H | (d) CS=1000H, IP=2000H |
| --- | --- | --- | --- |
| 1A000H + B800H = 1AB800H | 23000H + 1000H = 24000H | 23000H + 1A00H = 24A00H | 10000H + 2000H = 12000H |

Question 2: Which register is offset address for string destination?

Answer: DI (Destination Index)

Question 3: Stack memory is addressed by ______ segment plus ______ offset.

Answer: SS (Stack Segment) + SP or BP

Question 4: If BP addresses memory, which segment contains data?

Answer: SS (Stack Segment)

Question 5: Determine memory location addressed

| Pair | Calculation | Physical Address |
| --- | --- | --- |
| SS=2900H, SP=3A00H | 29000H + 3A00H = | 2CA00H |
| DS=1000H, SI=1002H | 10000H + 1002H = | 11002H |
| DS=1000H, DX=1000H | 10000H + 1000H = | 11000H |
| DS=1000H, DI=2000H | 10000H + 2000H = | 12000H |

Question 6: What is wrong with MOV BL, CX?

Answer: Size mismatch – BL is 8-bit, CX is 16-bit.

Question 7: 8085: MOV A, B then ADD C – step by step

Step 1: MOV A, B → A = B (flags unchanged)
Step 2: ADD C → A = B + C
Flags affected: S, Z, AC, P, CY

Question 8: Load 0x7F, add 0x01, branch if carry

assembly

MVI A, 7FHADI 01HJC LABEL_OVERFLOWHLTLABEL_OVERFLOW: HLT

Question 9: CALL and RET implementation

CALL: Pushes (PC+3) onto stack, then jumps to subroutine.

RET: Pops address from stack back to PC.

Example:

assembly

CALL ADD_BC...ADD_BC: MOV A, B        ADD C        RET

Question 10: Set bit 3 of A without affecting other bits

assembly

ORI 08H   ; 0000 1000 binary

Question 11: Conditional move (if Z=1, D→E else E→D)

assembly

JZ MOVE_D_TO_EMOV D, EJMP DONEMOVE_D_TO_E: MOV E, DDONE: HLT

Question 12: Clear carry only if bit 7 of A = 0

assembly

RLC        ; move bit 7 to CYJNC CLEAR  ; if CY=0 (bit7=0), jumpRETCLEAR: CMC ; clear carry (assuming it was 1? Actually CMC complements); Better:ANA A      ; set flags based on AJP SKIP    ; if bit7=0, sign flag=0CMC        ; complement carrySKIP: ...

Question 13: Difference between STA addr and STAX B

| Instruction | Addressing Mode | Operation |
| --- | --- | --- |
| STA addr | Direct | Stores A at fixed 16-bit address |
| STAX B | Register Indirect | Stores A at address in BC register pair |

Question 14: Conditional return when carry is set – mnemonic?

Answer: RC (Return if Carry)

Question 15: Which registers form SP? How modified by PUSH/POP?

SP = Stack Pointer (16-bit)

PUSH: SP ← SP – 2

POP: SP ← SP + 2

Question 16: How does DAD affect carry flag?

Answer: DAD adds 16-bit register pair to HL. CY flag is set if there is a carry from the 16-bit addition.

Question 17: Sequence to load A with byte at address in HL?

Answer: MOV A, M

Question 18: CPI vs CMP difference

| Instruction | Operand | Addressing |
| --- | --- | --- |
| CPI d8 | Immediate 8-bit | Immediate |
| CMP R/M | Register or Memory | Register / Indirect |

Question 19: Name an instruction that affects only the carry flag

Answer: CMC (Complement Carry) or STC (Set Carry)

Question 20: What does RRC do to accumulator bits?

RRC (Rotate Right through Carry):

Bit 0 → CY

CY → Bit 7

All bits shift right by one position

Question 21: How to add immediate 16-bit value to HL?

Answer: Load the value into another register pair, then use DAD.

assembly

LXI D, 1234HDAD D

Question 22: TRAP interrupt – maskable? Priority?

Answer: Non-maskable, Highest priority

Question 23: ALE signal purpose?

Answer: Address Latch Enable – latches lower-order address from multiplexed AD0–AD7 lines.

Question 24: Program Counter (PC) function?

Answer: Holds address of next instruction to be executed.

Question 25: Stack Pointer (SP) function?

Answer: Points to top of stack (LIFO memory area).

Question 26: Accumulator (A) function?

Answer: 8-bit register for arithmetic, logical, and data transfer operations.

Question 27: Temporary registers W and Z?

Answer: Internal, programmer-inaccessible registers used during instruction execution.

Question 28: Which registers are inaccessible to programmer in 8085?

Answer: W, Z, and Instruction Register

Question 29: Memory-mapped I/O advantage?

Answer: Uses rich instruction set (MOV, ADD, etc.) without special IN/OUT.

Question 30: Isolated I/O advantage?

Answer: Separate address space, does not consume memory addresses.

If you want me to:

Create a printable answer sheet

Solve the ADD BX, DX flag calculation

Explain any answer in more detail

Just tell me which one!